Supplementary Material 1

Mathematical Foundations of EcoBound

Ecological Change Rate (ECR) and Trend Detection

EcoBound begins by quantifying ecological dynamics through robust trend analysis across multi-temporal, multi-dimensional temporal raster datasets. We propose the Ecological Change Rate (ECR), calculated using the Theil-Sen Median slope estimator due to its robustness against outliers and non-normal data distributions. Given a temporal raster series {Yt}n t=1, representing ecological indicators such as vegetation indices (NDVI) or binary ecological states (e.g., grassland vs. other land-cover types), the Theil-Sen estimator calculates the slope (β) as Equation S1-1.

β=MedianYj-Yij-i，∀i<j,j∈[1,n]    Equation S1-1

To statistically evaluate the significance of these trends, we apply the Mann-Kendall non-parametric trend test, which calculates a standardized test statistic (Z) as Equation S1-2. The Z follows a normal distribution, where the absolute values of Z greater than 1.65, 1.96, and 2.58 indicate statistical significance at the 90%, 95%, and 99% confidence levels, respectively.

Z=S-1VarS,S>00,           S=0S+1Var(S),S<0    Equation S1-2

where S is the Mann-Kendall statistic calculated by Equation S1-3.

S=k=1n-1j=k+1nsgn(Yj-Yk), sgnx=1,  x>00,  x=0-1,  x<0    Equation S1-3

These metrics provide clear quantitative insights into the ecological resilience trajectory—whether ecosystems are recovering or degrading—forming the essential foundation for subsequent analysis.

Trend Detection Identification of Dominant Drivers (GeoDetector Analysis)

After establishing ecological change trends, EcoBound employs GeoDetector techniques (Song et al., 2020) to identify dominant environmental drivers and their interactions that significantly influence the ECR. The GeoDetector comprises two core modules used by EcoBound: factor detection and interaction detection.

Factor Detector quantifies the explanatory power (q-statistic) of a single environmental variable (X) influencing the spatial heterogeneity of the ECR (Y) as Equation S1-4.

q=1-h=1LNhσh2Nσ2, q∈[0,1]    Equation S1-4

where N and σ2 are the total number of pixels and overall variance of raster Y, respectively, and Nh and σ2 hrepresent pixel number and variance within class h of factor X. Classes h are defined by spatial stratification methods described in detail in Supplementary Material 1. Higher q indicates stronger explanatory power of the environmental factor.

Interaction Detector further explores combined effects between two environmental factors (X1 and X2), by comparing the joint explanatory power q(X1∩X2) (Figure S1-1) with the individual qX1 and qX2. Interaction types are classified into five categories:

- Nonlinear Weaken: q(X1∩X2) < min(qX1, qX2), suggesting a suppressive interaction.

- Uni-variable Weaken: min(qX1, qX2) < q(X1∩X2) < max(qX1, qX2), indicating marginal or inconclusive interactions

- Bi-variable Enhance: max(qX1, qX2) < q(X1∩X2) < qX1 + qX2, reflecting synergistic effects of both factors.

- Independent: q(X1∩X2) = qX1 + qX2, indicating additive but non-interacting effects.

- Nonlinear Enhance: qX1 + qX2 < q(X1∩X2), revealing strong synergistic interaction beyond additivity

Figure S1-1. Visualization of the combined stratification of explanatory variables X1 and X2 (denoted as X1 ∩ X2,).

While this section details the mathematical foundations of the Factor Detector and Interaction Detector, EcoBound also includes two additional modules—Risk Detector and Ecological Detector—as part of its full GeoDetector-based analytical suite. The mathematical foundation of all four detectors is fully consistent with the original GeoDetector framework proposed by Song et al. (2020) [doi:10.1080/15481603.2020.1760434].

Advanced Ecological Risk Detection

To capture nonlinear ecological transitions and early-warning signals along continuous environmental gradients, EcoBound implements an Advanced Ecological Risk Detector. This module combines bin-based statistical envelopes with moving average smoothing techniques to highlight both localized fluctuations and broad-scale directional trends in ecological responses. Together, these methods enable the detection of potential thresholds and zones of ecological instability prior to formal model-based threshold fitting.

Binning and statistical envelopes

EcoBound stratifies all spatial pixels into 𝐶1 = 100 equal-width intervals based on the value range of the explanatory variable 𝑋 (e.g., precipitation or temperature). This enables a fine-grained analysis of how the ecological response variable Y behaves across the environmental gradient. For each interval 𝑖, EcoBound calculates the mean (Yi) and standard deviation (𝜎𝑖) of Y across all spatial pixels whose X values fall within that range.

To characterize the variability of ecological responses within each interval, EcoBound constructs statistical envelope bands using a default multiplier 𝐶2 = 1.5 times the standard deviation. The upper and lower bounds for interval i are defined as Equations S1-5 and S1-6.

Upper Bandi=Yi+C2σi     Equation S1-5

Lower Bandi=Yi-C2σi     Equation S1-6

These statistical envelopes, conceptually analogous to ecological confidence intervals, define the expected range of Y under typical environmental conditions. Values falling outside the envelope suggest potential ecological deviations, transitions, or early signs of threshold behavior.

Moving averages for trend detection

To capture both localized and broad-scale trends in the ecological response variable Y along the environmental gradient X, EcoBound computes two simple moving averages (SMA) using different window sizes. These SMAs provide a smoothed representation of Y, highlighting short-term fluctuations as well as longer-term directional shifts.

The short-range moving average, denoted as SMAC3, is computed using a rolling window of 10 intervals by default (C3 = 10% × C1). This parameter is user-adjustable and determines the sensitivity of the indicator to short-term fluctuations in Y, enabling the detection of rapid ecological responses to small-scale variations in X. In contrast, the long-range moving average SMAC4 is calculated over a default window of 20 intervals (C4 = 20% × C1), smoothing out short-term noise to reveal broader-scale trends in ecological change across the full range of X.

MACD-based threshold analysis

To further enhance the detection of nonlinear ecological transitions, EcoBound incorporates a trend momentum indicator based on the Moving Average Convergence–Divergence (MACD) method. Originally developed in financial time series analysis, MACD is adapted here to quantify the rate and direction of change in the ecological response variable Y along stratified intervals of the explanatory variable X.

At the core of MACD is the Exponential Moving Average (EMA), which assigns greater weight to more recent observations. Compared to simple moving averages, EMA is more responsive to short-term shifts, making it particularly effective in capturing early ecological regime changes. The EMA at interval i, given a window size w, is defined recursively as Equation S1-7.

EMAwi=αYi+1-αEMAwi-1, α=2/(w+1)    Equation S1-7

where Yi is the mean of Y within interval i, and α is the smoothing factor that determines the weighting decay. For initialization, EMAw(1) is set to Y1.

EcoBound computes two EMAs of Yi: a short-term EMA with a default window of 12 intervals (C5 = 12% × C1), and a long-term EMA with a default window of 26 intervals (C6 = 26% × C1). The MACD index is defined as the difference between these two EMAs (Equation S1-8).

MACDi=EMAw=12i-EMAw=26i    Equation S1-8

To reduce noise and identify sustained shifts, a signal line is then calculated as an EMA of the MACD sequence over a default span of 9 intervals (C7 = 9% × C1, Equation S1-9).

Signali=EMA9(MACDi)    Equation S1-9

Finally, the MACD histogram is computed as the difference between the MACD line and the signal line (Equation S1-10).

Histogrami=MACDi-Signali    Equation S1-10

This histogram captures the momentum of ecological change. Positive values indicate increasing rates of ecological recovery or intensification, while negative values indicate declining trends or degradation. Intervals with large absolute histogram values, particularly those where MACD and signal lines cross, are flagged as candidate zones of ecological thresholds or instability. These flagged intervals are carried forward to the next stage for formal threshold modeling and spatial boundary delineation.

Risk zone classification based on MACD momentum

In addition to modeling ecological thresholds through logistic response functions, EcoBound incorporates a complementary module for quantifying the strength of ecological response dynamics. This is achieved by analyzing the momentum of change using MACD histograms. The resulting values reflect the degree of acceleration or deceleration in the ecological variable Y along the environmental gradient X and can be spatialized as continuous risk surfaces.

Dynamic baseline threshold τ

To establish data-driven reference point for interpreting a neutral momentum baseline, EcoBound calculates a dynamic threshold τ as the absolute weighted mean of MACD histogram values across all intervals (Equation S1-11).

μhist=i=1C1Histogrami×nii=1C1ni, τ = | 𝜇ℎ𝑖𝑠𝑡 |    Equation S1-11

where Histogrami is the MACD histogram value (Equation S1-10) and niis the number of spatial pixels in interval i. This threshold provides a neutral baseline that adapts to the intrinsic variability of the dataset.

Risk value calculation

To quantify the strength of ecological change along the environmental gradient, EcoBound calculates a continuous risk value for each interval i as the absolute difference between the MACD and signal line (Equation S1-12).

Riski=|MACDi-Signali|    Equation S1-12

This metric reflects the momentum intensity, or the rate at which the ecological response variable Y is accelerating or decelerating in response to the explanatory variable X. High values of Riski indicate that Y is undergoing rapid, potentially nonlinear changes—either increasing or decreasing—relative to its baseline trend. Such abrupt shifts are often early indicators of ecological instability or transitional dynamics, and therefore highlight zones where the system may be approaching a critical threshold.

From an ecological perspective, rapid positive changes in Y are not always desirable. While increasing trends in indicators such as vegetation cover, productivity, or biodiversity may appear beneficial, excessive or accelerated gains can signal an imbalance or overcompensation within the system. This may occur in contexts such as overshooting resource use, invasive species proliferation, or ecosystem responses pushed beyond natural resilience thresholds. As a result, EcoBound flags both unusually fast increases and decreases in Y as potential high-risk zones, emphasizing system stability over short-term improvement.

Optional classification into risk zones

While the risk value is continuous, users may optionally classify each interval into one of three qualitative risk zones based on its deviation from the baseline threshold τ, using a user-defined sensitivity coefficient k (default: 1). The classification is defined as Equation S1-13.

High Risk: Riski>k× τLow Risk: Riski<Signali-k× τMedium Risk: -k× τ≤Riski≤k× τ     Equation S1-13

- High risk: A high-risk interval indicates that Y is undergoing a significantly accelerated upward trend relative to its local baseline (Signali). According to resilience theory and early warning signal frameworks, such abrupt increases may reflect reduced system resilience or the approach of a critical ecological threshold. Even when Y represents a desirable attribute (e.g., vegetation cover, productivity, biodiversity), excessive or nonlinear growth can signal instability or unsustainable feedbacks. These conditions often arise during the r phase of the adaptive cycle or in systems overshooting their natural carrying capacity. Without timely intervention, such zones may be prone to regime shifts, collapse, or resource exhaustion.

- Low risk: A low-risk interval reflects a significantly downward trend in Y beyond the expected dynamic range, which can signify either recovery or decline depending on the ecological context. For negative indicators (e.g., pollutant concentration, degradation index), this trend may indicate successful mitigation or self-regulation. For positive attributes, however, it may suggest loss of function or ecosystem stress. Under the lens of the panarchy framework, these areas may be transitioning into a reorganization or renewal phase (α phase), offering a potential window for restoration. Continuous management is still essential to consolidate these gains or prevent further degradation.

- Medium risk: Intervals falling within ± kτ are considered to be within the range of expected ecological variability. These areas exhibit trends in Y that are consistent with background dynamics and may reflect resilient states or zones of adaptive equilibrium. According to the intermediate disturbance hypothesis, such moderate fluctuation regimes can sustain higher biodiversity and ecological function by balancing disturbance and recovery. While not immediately concerning, these zones should be closely monitored for signs of tipping points, directional drift, or cumulative stress that could shift them into high- or low-risk categories.

Spatialization and pixel-level synthesis of global risk index

The computed risk values Riski (Equation S1-12) are assigned to each spatial pixel based on its corresponding interval along the environmental value X, resulting in a continuous risk raster for each explanatory variable. These rasters capture the spatially explicit momentum strength of ecological change, allowing fine-scale identification of areas experiencing rapid responses to environmental drivers.

Robust per-layer normalization

Because the numerical range of Riski can differ among variables, we rescale every risk raster using a robust upper reference. For each j we compute the 98th percentile Tj over valid pixels and define by Equation S1-14a.

Riskj*p=minRiskjpTj+ε,1ϵ [0,1]    Equation S1-14a

with a small ε to avoid division by zero. The set {Tj} is reported for reproducibility.

Pixel-wise weighted synthesis

Explanatory power weights qj​ are taken from GeoDetector Qmax results. Optionally, only rows with Significant = True are retained (significance switch). Let J(p) be the subset of variables for which Riskj*p is valid (non-NoData). We compute S1-14b.

Sp=j∈JpqjRiskj*p,  Wp=j∈Jpqj    Equation S1-14b

Our Global Ecological Risk Index (GERI) is the pixel-wise weighted average, defined as Equation S1-14c, where p denotes a pixel location.

GERIp=SpWp， Wp>00,         Wp=0    Equation S1-14c, per-pixel normalized

This spatially explicit computation preserves the local heterogeneity of risk responses while integrating across multiple drivers. The resulting GERI raster enables the delineation of composite high-risk zones, which can be directly used for ecological risk mapping, conservation prioritization, and adaptive management planning.

EcoBound: A data-driven algorithm for ecological threshold and boundary detection

Conceptual Background and H₀ Hypothesis

EcoBound formulates ecological threshold detection as a nonparametric hypothesis testing problem grounded in variance decomposition. The goal is to determine whether splitting the ecological response variable Y based on candidate thresholds of the environmental gradient X significantly reduces within-group variance.

Null hypothesis (H₀)

Y shows no significant differentiation along X. Formally:

c=1Cncσc2≈Nσ2     Equation S1-15

where nc and σc2 are the sample size and variance within bin c, and N, σ2 are the total sample size and variance across all data.

Alternative hypothesis (H₁)

There exists a threshold along X that partitions Y into two groups with significantly reduced within-group variance, i.e.,

∆V=1-c=1Cncσc2Nσ2 > 0     Equation S1-16

The observed ΔV is then evaluated against a null distribution obtained via random permutation of Y, while X remains fixed. If ΔV exceeds the 95th percentile of this distribution, we reject H₀ and consider the detected threshold statistically significant.

Mathematical Framework of Ecobound

Primary segmentation of environmental gradient

Let X ∈ Rn  be the spatial raster of an environmental variable (e.g., precipitation, elevation). We divide the domain of X into C equal-width intervals (Equation S1-17).

Bins=I1, I2,…,IC,     Ii=[xi,xi+1)     Equation S1-17

For each bin Ic, we calculate:

- The number of valid raster cells nc

- The mean ecological response value Rc, where:

Rc=i∈IcYinc     Equation S1-18

This results in a sequence of mean response values R = {R1, ... , RC} and a sequence of weight values w = {n1, ... , nC} over the environmental gradient for the input of the next step.

Entropy-based Threshold Scanning

Discretization of ecological response states

We normalize and discretize the response values R into a finite set of ecological states using Equation S1-19.

Ri''=minRi-minRmaxR-minR+10-12×B,B-1, i=1, …, C    Equation S1-19

where:

- B is the number of discrete states, a parameter default = 30 inputted by users, determining the computing resolution and time.

Weighted frequency statistics

For each candidate threshold index k ∈ {1, ..., C−1}, the sequence R′′ is split into two groups: a left segment containing bins i < k, and a right segment containing bins i ≥ k. For each side, we compute the weighted frequency of each ecological state bin b ∈ {0, ..., B−1} (Equation S1-20).

nbLeftk=i<kRi''= bni,  nbRightk=i≥kRi''= bni     Equation S1-20

These counts are then normalized to obtain probability distributions (Equation S1-21).

PbLeftk=nbLeftkbnbLeftk   ,    PbRight(k)=nbRightkbnbRightk     Equation S1-21

A global reference distribution is also computed across all bins (Equation S1-22).

nbAll=nbLeft+nbRight,    PbAll=nbAllbnbAll     Equation S1-22

Entropy computation

For each candidate split 𝑘, we compute Shannon entropy for both partitions and the global distribution (Equation S1-23).

HLeftk=-bPbLeftklnPbLeft(k),  HRightk=-bPbRightklnPbRight(k),

Hall=-bPbAlllnPbAll,     Equation S1-23

The information gain (structural separation) induced by the partition is

For the patched implementation released in EcoBound v3.30, the threshold score was reformulated as a weighted information-gain criterion (Equation S1-24):

IGweighted(k)=Hall-NL(k)NHL(k)+NR(k)NHR(k)     Equation S1- SEQ Equation \* ARABIC 24

where NL(k)and NR(k) are the total weights (number of cells) of the left and right partitions and N=NL(k)+NR(k).

This weighted formulation reduces edge bias by accounting for the relative contribution of the two partitions to the full distribution and is therefore used in the current public release. This measures how much internal structure is revealed by partitioning the response curve at position 𝑘.

Optimal threshold selection

The ecological threshold is selected as the position that maximizes the information gain (Equation S1-25).

k*=argmax∆Hk     Equation S1-25

The final threshold value is the environmental midpoint (Equation S1-17) of the 𝑘∗-th primary bin (Equation S1-26).

Tentropy= center(Ik*)     Equation S1-26

This approach allows EcoBound to detect sharp or subtle transitions in ecological responses without requiring model fitting or monotonic assumptions, ensuring robustness across complex nonlinear systems.

Illustrative example

To demystify the entropy-based threshold detection, we present a simplified example with five environmental bins and known ecological responses. Let the ecological response means be:

R = {1, 2, 3, 4, 5} with weights (number of pixel counts) w = {6, 7, 8, 9, 10}

Taking R to Equation S1-19 gets R'' = {0, 1, 2, 3, 4}, as box S1-1.

Box S1-1. Discretization of ecological response means R→R′′ using Python-style logic.

# Inputs R = np.array([1, 2, 3, 4, 5], dtype=float) # Ecological response means B = 5 # Number of discrete states # Normalize and discretize to integer states R_min = R.min() R_max = R.max() R_range = R_max - R_min + 1e-12 # Small epsilon to avoid zero division R_normalized = (R - R_min) / R_range R_discrete = np.floor(R_normalized * B).astype(int) # Ensure upper bound does not exceed B-1 R_discrete = np.minimum(R_discrete, B - 1) # Output # R_discrete = [0, 1, 2, 3, 4]

Suppose we test a candidate threshold k=3 (Equation S1-20), splitting the bins into:

- Left: i = 1, 2 (with R′′ = {0, 1} and w = {6, 7})

- Right: i = 3, 4, 5 (with R′′ = {2, 3, 4} and w = {8, 9, 10})

Take them to Equation S1-21:

- Left: nLeft 0 = 6, nLeft 1 = 7 ⇒ PLeft 0 = 6/(6+7) = 6/13, PLeft 1 = 7/(6+7) = 7/13

- Right: nRight 2 = 8, nRight 3 = 9, nRight 4 = 10 ⇒ PRight 2 = 8/(8+9+10) = 8/27, PRight 3 = 9/27, PRight 4 = 10/27

This example illustrates the two-stage binning logic behind EcoBound: the first discretization segments the environmental gradient, and the second projects ecological responses into interpretable state bins, enabling data-driven detection of ecological transitions.

Variance Reduction

While entropy-based scanning identifies the location of ecological inflection points, an independent statistical test is required to confirm that the threshold meaningfully separates the response variable 𝑌. To this end, EcoBound employs a variance reduction index (VR), which quantifies the drop in within-group variance induced by splitting 𝑌 at the candidate threshold.

Let Tentropy denote the threshold obtained from entropy scanning (Equation S1-26). The total variance of the response variable Y is calculated over all valid raster cells (Equation S1-27).

σ2=1N-1i=1N(Yi-Y)2     Equation S1-27

EcoBound then directly splits the full-resolution raster Y based on whether the corresponding environmental value Xi is less than or greater than Tentropy resulting in two groups:

- Group 1: Xi1 < Tentropy

- Group 2: Xi2 ≥ Tentropy

Let:

- n1 and σ12 be the number of raster cells and sample variance of 𝑌 in Group 1

- n2 and σ22 be the number of raster cells and sample variance of 𝑌 in Group 2

These are computed directly from the individual raster pixels, independent of any prior binning or summarization.

The VR is then calculated as Equation S1-28.

VR=1-n1σ12+n2σ22Nσ2     Equation S1-28

This statistic VR measures the proportion of total variance explained by the threshold-induced split. Unlike Geodetector's q-statistic, which relies on predefined spatial strata, ΔV uses data-driven stratification derived from the explanatory variable 𝑋, making it more adaptive to ecological gradients.

Ecologically, VR can also be interpreted as a quantitative index of how strongly the environmental gradient 𝑋 controls or drives the ecological response 𝑌. Higher VR values indicate greater effect sizes and clearer ecological separability across the threshold Tentropy.

Significance testing via permutation

To evaluate whether the observed VRobs is statistically significant, EcoBound performs a nonparametric permutation test:

- The values of Y are randomly permuted, while the values of X remain fixed.

- For each permutation, a new VRr is computed based on the same threshold Tentropy.

- After R repetitions (e.g., 1000), the empirical distribution of VRnull is used to compute a one-tailed p-value (Equation S1-29).

p= 1Rr=1RII(VRr≥VRobs)     Equation S1-29

where II( ) is the indicator function.

If p<0.05, the null hypothesis is rejected, and the threshold is considered statistically meaningful.

Natural Geographic Boundary Detection and Spatial Delineation

After determining the optimal ecological threshold Tentropy for each environmental variable Xi, EcoBound proceeds to delineate spatially explicit natural geographic boundaries that correspond to this break value. This step bridges the gap between statistical threshold detection and geographic pattern identification, ensuring that the detected ecological transitions are not only meaningful in the attribute space but also coherent in the spatial domain.

EcoBound first divides the entire study area into two initial value-based groups:

- Group 1: Xi1 < Tentropy

- Group 2: Xi2 ≥ Tentropy

These groups correspond to global partitions based solely on environmental values, without considering spatial contiguity. However, such segmentation may result in scattered or isolated pixel patches, which hinders the geographic interpretability of ecological boundaries. To address this issue, EcoBound applies a connected-component analysis using a moving window approach to identify spatially continuous subregions within each group. Specifically, all pixels in a subregion Ck (where k = 1, 2) must satisfy the spatial connectivity condition (Equation S1-30):

∀pi, pj∈Ck,dpi,pj<∞    Equation S1-30

where pi and pj denote any pair of pixels within the same subregion, and d(pi, pj) represents the spatial distance. This condition ensures that each subregion forms a spatially connected cluster.

As multiple candidate subregion pairs may exist, EcoBound selects the combination of (C1, C2) that minimizes the number of spatial outliers—pixels that are inconsistent with their assigned group based on the threshold. This optimization is formalized as (Equation S1-31):

min[n(pC1 > Tentropy) + n((pC2 < Tentropy)]    Equation S1-31

where n(pC1 > Tentropy) and n((pC2 < Tentropy) represent the number of pixels in C1 > Tentropy and the number of pixels in C2 < Tentropy, respectively. By minimizing these so-called "exclave" pixels, EcoBound enhances the spatial coherence and ecological validity of the boundary.

Finally, the transition zone between the optimal connected subregions C1  and C2 is extracted and vectorized into a natural geographic boundary shapefile. This boundary represents a geographically interpretable, entropy-informed transition zone that delineates ecologically distinct regions in the landscape. The output shapefile serves as a critical input for regional zoning, conservation planning, and ecological monitoring.